1. Problem Statement: 8-bit BCD Adder

Design code

module bcd_adder(

input [7:0] A,

input [7:0] B,

input Cin,

output [7:0] S,

output Cout

);

logic [3:0] lsum, hsum;

logic lcarry, hcarry;

assign {lcarry, lsum} = A[3:0] + B[3:0] + Cin;

assign {hcarry, hsum} = A[7:4] + B[7:4] + (lsum > 4'd9);

assign S = { (hsum > 4'd9) ? (hsum + 4'd6) : hsum,

(lsum > 4'd9) ? (lsum + 4'd6) : lsum };

assign Cout = (hsum > 4'd9) || hcarry;

endmodule

RTL

Testbenh:

module testbench;

reg [7:0] A;

reg [7:0] B;

reg Cin;

wire [7:0] S;

wire Cout;

bcd_adder uut (

.A(A),

.B(B),

.Cin(Cin),

.S(S),

.Cout(Cout)

);

initial begin

$monitor("Time: %0d | A=%d B=%d Cin=%b | S=%d Cout=%b", $time, A, B, Cin, S, Cout);

A = 8'd45; B = 8'd36; Cin = 0; #10;

A = 8'd99; B = 8'd99; Cin = 0; #10;

A = 8'd12; B = 8'd89; Cin = 1; #10;

A = 8'd50; B = 8'd50; Cin = 0; #10;

A = 8'd19; B = 8'd81; Cin = 0; #10;

$finish;

end

endmodule

wave form:

Synthesis reort:

TECHNOLOGY DIAGRAM

2.Problem Statement: FPGA Flow – Combinational & Sequential circuits

(a) Boolean Simplification

Design code:

module combinational_a (

input A, B, C, D,

output Y

);

assign Y = (~A & ~B & ~C & ~D) | (A & B & C) | (~A & B & C & ~D) | (A & B & D) | (B & C & D) | (~A);

endmodule

RTL:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________

TESTBENCH:

module testbench;

logic A, B, C, D;

logic Y;

combinational_a uut (

.A(A), .B(B), .C(C), .D(D), .Y(Y) );

initial begin

$display("Testbench for comb_function1");

$display("A B C D | Y");

A = 0; B = 0; C = 0; D = 0; #5;

$display("%b %b %b %b | %b", A, B, C, D, Y);

A = 1; B = 0; C = 1; D = 1; #5;

$display("%b %b %b %b | %b", A, B, C, D, Y);

A = 1; B = 1; C = 0; D = 1; #5;

$display("%b %b %b %b | %b", A, B, C, D, Y);

A = 0; B = 1; C = 1; D = 0; #5;

$display("%b %b %b %b | %b", A, B, C, D, Y);

A = 1; B = 1; C = 1; D = 1; #5;

$display("%b %b %b %b | %b", A, B, C, D, Y);

$display("Test complete.");

$finish;

end

endmodule

WAVEFORM:

TECHNOLOGY SCREENSHOT:

Synthesis REPORT:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(b)COMBINATIONAL_B

DESIGN CODE:

module combinational_b (

input A, B, C, D, E,

output Y

);

assign Y = (A & B & C) | (A & B & D) | (A & B & E) | (A & C & D) | (A & C & E) |

(A | D | E) | (B & C & D) | (B & C & E) | (B & D & E) | (C & D & E);

Endmodule

________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Testbench

module testbench;

logic A, B, C, D, E;

logic Y;

combinational_b uut (

.A(A), .B(B), .C(C), .D(D), .E(E), .Y(Y) );

initial begin

$display("Testbench for comb_function2");

$display("A B C D E | Y");

// Test cases

A = 0; B = 0; C = 0; D = 0; E = 0; #5; $display("%b %b %b %b %b | %b", A, B, C, D, E, Y);

A = 1; B = 0; C = 1; D = 1; E = 0; #5; $display("%b %b %b %b %b | %b", A, B, C, D, E, Y);

A = 0; B = 1; C = 0; D = 1; E = 1; #5; $display("%b %b %b %b %b | %b", A, B, C, D, E, Y);

A = 1; B = 1; C = 1; D = 0; E = 1; #5; $display("%b %b %b %b %b | %b", A, B, C, D, E, Y);

A = 1; B = 1; C = 1; D = 1; E = 1; #5; $display("%b %b %b %b %b | %b", A, B, C, D, E, Y);

$display("Test complete.");

$finish;

end

endmodule

RTL:

WAVEFORM:

TECNOLOGY SCREENSHOT:

SYNTHESIS REPORT:

________________________________________________________________________________________________________________________________________________________________________________________________________________________________

(c): Two-Output Function

DESIGN CODE:

module two_output_function(input A, B, C, D, output Y1, Y2);

assign Y1 = A | (B & C) | ~D;

assign Y2 = A & B & ~C & D;

endmodule

TESTBENCH:

module testbench;

reg A, B, C, D;

wire Y1, Y2;

two_output_function uut (.A(A), .B(B), .C(C), .D(D), .Y1(Y1), .Y2(Y2));

initial begin

$monitor("A=%b, B=%b, C=%b, D=%b -> Y1=%b, Y2=%b", A, B, C, D, Y1, Y2);

A = 0; B = 0; C = 0; D = 0;

#10;

A = 1; B = 0; C = 1; D = 0;

#10;

A = 0; B = 1; C = 0; D = 1;

#10;

A = 1; B = 1; C = 1; D = 1;

#10;

A = 0; B = 1; C = 1; D = 0;

#10;

$finish;

end

endmodule

RTL:

WAVEFORM;

TECHNOLOGY SCREENSHOT:

SYMTHESIS  REPORT:

________________________________________________________________________________________________________________

(d) Prime Number and Divisibility Logic

Design Code :

module divisibility(input [3:0] A, output P, D);

assign P = (A == 4'd2) | (A == 4'd3) | (A == 4'd5) |

(A == 4'd7) | (A == 4'd11) | (A == 4'd13);

assign D = (A % 3 == 0);

endmodule

TESTBENCH:

module testbench;

reg [3:0] A;

wire P, D;

divisibility uut (.A(A), .P(P), .D(D));

initial begin

$monitor("A=%b (%d) -> P=%b (Prime), D=%b (Divisible by 3)", A, A, P, D);

A = 4'd0; #10;

A = 4'd3; #10;

A = 4'd5; #10;

A = 4'd9; #10;

A = 4'd14; #10;

$finish;

end

endmodule

RTL:

WAVEFORM:

TECNOLOGY SCREENSHOT:

SYNTHESIS REPORT:

(e): 4-Bit Priority Encoder

DESIGN CODE:

module priority_encoder_4(input [3:0] A, output logic [1:0] Y, output reg valid);

always @(*) begin

valid = (A != 0);

casez (A)

4'b1???: Y = 2'b11;

4'b01??: Y = 2'b10;

4'b001?: Y = 2'b01;

4'b0001: Y = 2'b00;

default: Y = 2'b00;

endcase

end

endmodule

TESTBENCH

module testbench;

reg [3:0] A;

wire [1:0] Y;

wire valid;

priority_encoder_4 uut (.A(A), .Y(Y), .valid(valid));

initial begin

$monitor("A=%b -> Y=%b, Valid=%b", A, Y, valid);

A = 4'b0000; #10;

A = 4'b0001; #10;

A = 4'b1000; #10;

A = 4'b1010; #10;

A = 4'b0110; #10;

$finish;

end

endmodule

RTL: WAVEFORM:

TECHNOLOGY SCREENSHOT:

SYNTHESIS REPORT:

(f) 8-Bit Priority Encode

module priority_encoder_8(input [7:0] A, output logic [2:0] Y, output reg valid);

always @(*) begin

valid = (A != 0);

casez (A)

8'b1???????: Y = 3'b111;

8'b01??????: Y = 3'b110;

8'b001?????: Y = 3'b101;

8'b0001????: Y = 3'b100;

8'b00001???: Y = 3'b011;

8'b000001??: Y = 3'b010;

8'b0000001?: Y = 3'b001;

8'b00000001: Y = 3'b000;

default: Y = 3'b000;

endcase

end

endmodule

TESTBENCH:

module testbench;

reg [7:0] A;

wire [2:0] Y;

wire valid;

priority_encoder_8 uut (.A(A), .Y(Y), .valid(valid));

initial begin

$monitor("A=%b -> Y=%b, Valid=%b", A, Y, valid);

A = 8'b00000000; #10;

A = 8'b00000001; #10;

A = 8'b10000000; #10;

A = 8'b01001000; #10;

A = 8'b00101100; #10;

$finish;

end

endmodule

RTL:

WAVEFORM: TECHNOLOGY SCREESHOT

SYNTHESIS  REPORT:

(i) FSM

Design code :

module fsm(input clk, reset, X, output logic [1:0] state);

parameter S0 = 2'b00, S1 = 2'b01, S2 = 2'b10;

always @(posedge clk or posedge reset) begin

if (reset)

state <= S0;

end

else begin

case (state)

S0: state <= X ? S1 : S0;

S1: state <= X ? S2 : S0;

S2: state <= X ? S2 : S1;

endcase

end

endmodule

TESTBENCH:

module testbench;

reg clk, reset, X;

wire [1:0] state;

fsm uut (.clk(clk), .reset(reset), .X(X), .state(state));

initial begin

clk = 0;

forever #5 clk = ~clk;

end

initial begin

$monitor("Time=%0t | Reset=%b, X=%b -> State=%b", $time, reset, X, state);

reset = 1; X = 0; #10;

reset = 0; X = 1; #10;

X = 1; #10;

X = 0; #10;

X = 0; #10;

$finish;

end

endmodule

RTL:

WAVEFORM:

TECHNOLOGY SCREENSHOT: SYNTHESIS REPORT:

(g) 3-to-8 Decoder

Design code;

module decoder_3to8(input [2:0] A, output logic [7:0] Y);

always @(*) begin

Y = 8'b00000000;

case (A)

3'b000: Y[0] = 1;

3'b001: Y[1] = 1;

3'b010: Y[2] = 1;

3'b011: Y[3] = 1;

3'b100: Y[4] = 1;

3'b101: Y[5] = 1;

3'b110: Y[6] = 1;

3'b111: Y[7] = 1;

endcase

end

endmodule

TESTBENCH:

module testbench;

reg [2:0] A;

wire [7:0] Y;

decoder_3to8 uut (.A(A), .Y(Y));

initial begin

$monitor("A=%b -> Y=%b", A, Y);

A = 3'b000; #10;

A = 3'b001; #10;

A = 3'b010; #10;

A = 3'b100; #10;

A = 3'b111; #10;

$finish;

end

endmodule

RTL:

WAVEFORM:TECHNOLOGY SCREENSHOT:

SYNTHESIS REPORT:

(h) A 4-Bit carry propagate adder(with no carry in)

Design code

module carry_propagate_adder_4bit_no_carry(input [3:0] A, B, output [3:0] Sum);

assign Sum = A + B;

endmodule

TESTBENCH:

module testbench;

reg [3:0] A, B;

wire [3:0] Sum;

carry_propagate_adder_4bit_no_carry uut (.A(A), .B(B), .Sum(Sum));

initial begin

$monitor("A=%b (%d), B=%b (%d) -> Sum=%b (%d)", A, A, B, B, Sum, Sum);

A = 4'b0001; B = 4'b0010; #10;

A = 4'b0101; B = 4'b0011; #10;

A = 4'b1001; B = 4'b0110; #10;

A = 4'b1111; B = 4'b0001; #10;

A = 4'b0000; B = 4'b1010; #10;

$finish;

end

endmodule

RTL:

WAVEFORM: TECHNOLOGY SCREENSHOT:

SYNTHESIS REPORT:

(j)The Gray code counter

DESIGN CODE:

module gray_code_counter (

input clk,

input reset,

output logic [2:0] gray_code

);

logic [2:0] binary_count;

always @(posedge clk or posedge reset) begin

if (reset) begin

binary_count <= 3'b000;

end

else

begin

binary_count <= binary_count + 1;

end

end

always @(*) begin

gray_code = binary_count ^ (binary_count >> 1);

end

endmodule

TESTBENCH:

module testbench;

reg clk;

reg reset;

wire [2:0] gray_code;

gray_code_counter uut (

.clk(clk),

.reset(reset),

.gray_code(gray_code)

);

always #5 clk = ~clk;

initial begin

clk = 0;

reset = 1;

#10;

reset = 0;

#100;

$stop;

end

initial begin

$monitor("Time=%0t | Reset=%b | Gray Code=%b", $time, reset, gray_code);

end

endmodule

RTL:
WAVEFORM: TECHNOLOGY SCREENSHOT:

SYNTHESIS REPORT